### PRESSURE SYSTEMS

## LOW PRESSURE AREA

## Also known as a cyclone or a depression,

- Wind flows counterclockwise and inward

- Pressure is lowest in the centre

- Expect poor weather

SECONDARY LOW

## smaller disturbance of cyclone nature

## forms within a main depression

TROUGH

- U shaped area of low pressure with higher pressure on either side.

RIDGE

- Similar to a trough but extends outward from a high with lower pressure on either side.

COL

- A neutral area between two highs and two lows.

#### HIGH PRESSURE AREA

#### also known as an anticyclone

- wind flows clockwise and outward

- pressure highest in centre

- expect fine to fair weather

WINDS

- Winds always flow from an area of high pressure to an area of low pressure.

FORCES AFFECTING HORIZONTAL AIR MOVEMENT

Pressure Gradient

- the rate of change of pressure over distance measured at right angles to the isobars

- it is steepest when isobars are close

- it determines wind velocity ( the closer the isobars the stronger the winds)

Coriolis Force

- Because the earth rotates beneath the atmosphere, air is deflected to the right in the Northern Hemisphere, until it flows parallel to the isobars.

Surface Friction

- Friction between the surface of the earth and the atmosphere will slow the movement of air. This in turn increases the angle at which the air crosses the isobars.

BUYS BALLOTS LAW

In the Northern Hemisphere, when you stand with your back to the wind, the low pressure area is to your left.

MET 403-04