## THUNDERSTORMS

A weather phenomena whose presence creates extremely serious hazards to flying.  They may be accompanied by:

- thunder

- lightning

- strong vertical drafts

- severe gusts and turbulence

- heavy rain

- hail

- micro/macrobursts

- tornadoes

- severe wind shear

It is a weather condition of which a pilot should be enormously respectful.

Thunderstorms must have the following requirements:

- Unstable air to high levels

- Lifting agent

- High moisture content

#### THE LIFE OF THUNDERSTORMS

Initial or Cumulus Stage

- Strong updrafts prevail (unstable air to high levels)

- Temperature is higher inside the cloud than the surrounding air

- Diameter ranges from 1 to 2 miles (but may become as large as 6 miles)

- Steep lapse rate

Mature Stage

##### Updrafts penetrate to great heights

- Downdrafts start in middle and lower level of cell

- Precipitation starts

- Stage usually lasts 15 to 30 minutes (may last as long as 60min).

Dissipating Stage

- -Downdraft occupies all but top of cloud where updrafts persist

- Rain starts to slow and stop

- Top of cloud frays into anvil shape.

TYPES

###### Air Mass

- usually form on hot summer days

- relatively easy to avoid

- form as a result of either convection or orgraphic lift

###### Frontal

- associated with cold fronts

- usually form in a line that may extend for hundreds of miles of cold front, known as LINE SQUALL

- can develop at warm front and may be embedded

THUNDERSTORMS HAZARDS

TURBULENCE

- could overstress aircraft or cause loss of control

- downdrafts as strong as 2000 feet per minute and updrafts as strong as 6000 feet per minute that can severely cripple aircraft

- strongest between 12,000 – 20,000 feet in mature stage

- can be experienced, in clear air up, to 20 miles away from severe cells

- can be severe in micro/macrobursts and at gust front.

WINDS

- danger of gusts up to 80kts with rapid changes in direction

- gust front generates strong, gusty winds near the surface which can change direction by 180° and gust up to 50 kts in seconds.

HAIL

- could cause serious structural damage

- can be encountered outside of cloud as it is thrown upward and outward by active cells.

ICING

- abundance of supercooled water droplets, will cause severe icing

- most severe during the mature stage.

##### LIGHTNING

- hampers vision for 30 – 50 seconds at a time

- greatest likelihood of strike at temperatures between

- -5C and + 5C)

- solid state circuitry is particularly vulnerable to strikes

- electrical circuitry may be disrupted

- possibility of igniting fuel vapour in fuel cells.

PRESSURE

- rapid changes in pressure cause unreliable altimeter readings.

# THUNDERSTORM AVOIDANCE

- If ou must fly past one, stay at least 15 miles away and pass to the right.

- A thunderstorm is an area of low pressure (Anti-clockwise and inward).

- You will encounter more favorable winds (tailwind) if you pass to the right.

- Never fly under a thunderstorm due to up/down drafts.

### BEST ADVISE – STAY ON THE GROUND

MET 403-10