ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL THREE | PO: PROPULSION |
| --- | --- |

PO/EO:	416.04b

ENABLING OBJECTIVE:	Explain the carburation system of an aircraft.

REFERENCE(S):	A.	A-CR-CCP-268/PH-001 Level Three Course Training Plan

Chapter 4, Pages 111-114.

B.	A-CR-CCP-268/PT-001 Level Three Handbook

Chapter 9, Section 4, Articles 28-34, Pages 9-12.

C.	A-CR-CCP-263/PT-001 From the Ground Up

Pages 58 and 61-63.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP Slides

C.	Diagrams (if available)

D.	Real/Model Engine (if available)

LEARNING AID(S):		A.	A-CR-CCP-268/PT-001 Level Three Handbook

TEACHING POINT(S):	A.	Carburetor construction

B.	Carburetor ice

METHOD OF INSTRUCTION:	Lecture

TIME:		2 of 2 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL THREE | PO: PROPULSION |
| --- | --- |

PO/EO:	416.04b

ENABLING OBJECTIVE:	Explain the carburation system of an aircraft.

REVIEW

PO/EO:	416.04a

ENABLING OBJECTIVE:	Explain the carburation system of an aircraft.

1.	Review weaknesses of the last EOs performance check; and/or

2.	Ask the following review question:

a.	What is the function of a carburetor?

Answer:	The function of the carburetor is to measure the correct quantity of gasoline, vapourize this fuel, mix it with air in the proper proportion and deliver the mixture to the cylinders.

INTRODUCTION

WHAT:	In this lesson you will learn about the carburation system of an aircraft.

WHY:	The carburetor is an important part of the engine and knowledge about it will enhance your knowledge of engines.

WHERE:	You can apply this knowledge when visiting an aerodrome for a facilities tour, at an air-show, during glider familiarization, or even while attending summer training.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	THE CARBURATION SYSTEM

5

MINs

1.	General:

a.	During the induction stroke of the four-stroke cycle, a charge of			vaporized fuel and air is drawn into the engine. This mixture of fuel 			and air is necessary for combustion.

b.	The necessary air is readily obtained from the surrounding				atmosphere, but the fuel must be metered to the engine to maintain 		the right proportion of fuel and air. The supply of this mixture to the

engine must also be metered to control the power output.

c.	The function of the carburetor is to measure the correct quantity of 			gasoline, vaporize this fuel, mix it with the air in the proper 				proportion and deliver the mixture to the cylinder.

2.	Carburetor Components:

The main components of a carburetor includes the following:

OHP # 1

a.	Air passage (choke tube) and choke valve;

b.	Throttle valve;

c.	Float chamber and float mechanism; and

d.	Main jet.

3.	How the Carburetor Works:

Supporting

Information

### in Italics

a.	As we just saw, the carburetor delivers a mixture of air and fuel to

the engine. It is therefore equipped with an intake for the air and

one for the fuel.

OHP # 1

b.	Fuel enters the carburetor at the float valve. It flows into the float

chamber where its level is controlled by a float that opens or closes

the float valve as it rises or falls. When the float rises to a

predetermined level, it shuts off the float valve. No additional fuel

can then enter the carburetor until fuel is used by the engine. The

float chamber is vented so 	that the pressure in the chamber

equalizes with the atmospheric pressure as the airplane climbs and

descends.

c.	The level of gasoline in the float chamber governs the level of

gasoline in the nozzle or main jet. The choke valve is used by the

pilot to control the flow of fuel to this nozzle, thus controlling the

mixture.

d.	Outside air passes through the carburetor air filter that is located at

the carburetor air intake in the front of the engine cowling. It moves

through the air passage to the carburetor. It is then drawn through

the choke tube, which is narrower than the air passage, where its

speed is increased. A low-pressure area is thereby created in the

choke tube. The reduced pressure around the main jet draws the

fuel, which is under atmospheric pressure, from the jet in the form

of a fine spray.

CONFIRMATION STAGE 1

1.	What are the main components of a carburetor?

Answer:	Air passage and choke valve, throttle valve, float chamber				and float mechanism, and main jet.

STAGE 2 	CARBURETOR ICE

18

MINs

1.	Carburetor Icing:

a.	Under certain moist atmospheric conditions, it is possible for ice to 			form in the induction system of an internal combustion engine. This

carburetor ice will gradually close off the flow of fuel to the engine.  			It is usually indicated by a loss of power or complete engine failure.

b.	A pilot flying in conditions conducive to carburetor icing will apply 			carburetor heat at regular intervals. The carburetor heat system 			warms the air entering the carburetor in order to prevent icing and 			can also clear ice that has accumulated in the carburetor. However,

when the carburetor air is heated, the engine cannot develop full

power and thus the carburetor heat is not engaged all the time.

2.	Causes:

OHP # 2

a.	Carburetor ice can be caused by two processes:

(i)	The drop in temperature as heat is taken from the air in				order to effect vaporization of the fuel; and

(ii)	Cooling due to the low-pressure area in the carburetor.

b.	Liquid gasoline must be changed into a vapour and mixed with air 			to become combustible. This process occurs in the carburetor.  			The heat that is required to change the liquid gasoline to a vapour 			is absorbed from the air that passes through the carburetor. The 			carburetor therefore, is in effect a miniature refrigerator.

c.	As the speed of the incoming air increases, the pressure decreases 		and has a cooling effect on the mixture. Moisture in this low-				pressure area will freeze and build up in the carburetor.

3.	Temperature:

a.	The temperature in the carburetor can drop as much as 30 degrees 		Celsius below the temperature of the incoming air. If this air 				contains large amounts of moisture, the cooling process will cause 			the water vapour to condense and possibly freeze on all surfaces of 		the carburetor. This may cut off the flow of mixture to the engine.

b.	Carburetor icing can occur at temperatures as high as 30 degrees			Celsius, although temperatures around 15 degrees should be the 			most suspect, especially on humid days.

CONFIRMATION STAGE 2

1.	What are the dangers of carburetor icing?

Answer:	Carburetor ice can cause a loss of power or complete

engine failure.

2.	What two processes cause carburetor ice?

Answer:	The drop in temperature as heat is taken from the air					in order to effect vaporization of the fuel; and, cooling due to				the low-pressure area in the carburetor.

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details - 	Each cadet will have to explain carburetor construction and carburetor

icing.

1.	Test knowledge with OHPs, including OHP #3. It may be either projected or photocopied and distributed.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you have learned about the carburetor system.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You can apply this knowledge when visiting an aerodrome for a facilities		tour, at an air-show, during glider familiarization, or even while attending 	summer training.

C.	Your next lesson is PO/EO 416.05. In this class you will learn about the supercharging and turbocharging.