ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL FOUR | PO: NAVIGATION |
| --- | --- |

PO/EO:	417.02a

ENABLING OBJECTIVE:	Identify Navigation Charts.

REFERENCE(S):	A.	A-CR-CCP-269/PH-001 Level Four Course Training Plan

Chapter 4, Pages 83-84.

B.	A-CR-CCP-269/PT-001 Level Four Handbook

Chapter 9, Section 2, Articles 1-13, Pages 1-4.

C.	A-CR-CCP-263/PT-001 From the Ground Up

Pages 173-174.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP Slides

# C.	Globe (if available)

LEARNING AID(S):		A.	A-CR-CCP-269/PT-001 Level Four Handbook

TEACHING POINT(S):	A.	Basic elements of map construction.

B.	The Lambert conformal conic projection

C.	The Mercator projection

METHOD OF INSTRUCTION:	Lecture

TIME:		1 of 2 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL FOUR | PO: NAVIGATION |
| --- | --- |

PO/EO:	417.02a

ENABLING OBJECTIVE:	Identify Navigation Charts.

REVIEW

PO/EO:	N/A

ENABLING OBJECTIVE:		N/A

INTRODUCTION

WHAT:	In this lesson you will learn about navigation charts.

WHY:	It is necessary to be able to read a VFR navigation chart in order to plot a flight.

WHERE:	You will have an opportunity to use this knowledge during the last Navigation class, where you will plot a flight plan.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	NAVIGATION CHARTS

5

MINs

1.	General:

a.	Some representation of the area over which an aircraft will pass is 			necessary to measure the direction and distance relative to the 			starting and the destination point of the flight.

b.	A map is a small-scale flat-surface representation of some portion 			of the earth's surface.  Usually a representation that is designed 			for plotting navigational information is called a chart, although 			the term map and chart are often used interchangeably.

c.	As the earth is a sphere, its surface cannot be represented on a			flat plane.  Since the surface of a sphere cannot be accurately			projected onto a map, a map must show the portion of the earth’s 			surface it represents with some distortion.

Demonstrate

using a globe.

d.	The only way of reproducing a true and undistorted picture of the			earth’s surface is to do so on the surface of a figure similar to the			earth, that is, on a sphere.  In other words, the only perfect maps in 			existence today are in the form of spheres, such as the common 			globes.  Globes are obviously too cumbersome for the cockpit.  For			practical purposes, maps are prepared on flat surfaces, but you 			must understand that no flat map can represent all or even a part of 		the earth's surface exactly in every detail.

e.	There are four basic elements in map construction. These are:

(i)	Areas;

(ii)	Shapes;

(iii)	Bearing; and

(iv)	Distances

f.	According to the particular purpose of a map, one or more of these			elements is preserved as nearly correctly as possible, with 				consequent unavoidable distortion in the remaining elements. The 			mathematical bases on which maps are constructed are termed 			projections.

g.	There are two principle types of chart projections used in air 				navigation charts:

(i)	Lambert Conformal Conic Projection

(ii)	Transverse Mercator Projection

15

MINs

2.	 The Lambert Conformal Conic Projection:

a.	The basic idea upon which the Lambert Conic Projection was			developed supposes a cone is superimposed over the surface of a 			sphere.

OHP #1

b.	On this type of map, the angles between meridians and parallels 			are the same on the map as they are on the ground.  The term 			conformal refers to this characteristic.  Scale must be the same 			along both meridians and parallels.

c.	The scale of distortion is practically uniform throughout the entire 			map sheet, the maximum distortion being not more than 0.5%.  			However, because the meridians on this type of map converge 			towards the poles, a straight line on the map will not have the same 		bearing when measured on different meridians.  Hence a straight 			line cannot be flown without changing heading at regular intervals.

OHP #2

3.	The Mercator Projection:

a.	The principle on which the Mercator Projection is based is that of a			cylinder which has its points of tangency at the Equator. Its				approximate form may be visualized by imagining a light at the 			centre of a globe that casts a shadow of the meridians and parallels 		on a cylinder of infinite length enclosing it. The shadows of the 			more northerly parallels of latitude are wider apart on the cylinder 			than they actually are on the sphere.

b.	The meridians on the cylinder are straight and parallel lines to 			infinity, whereas on the sphere they converge to meet in a point at 			the Poles. It is, in other words, as though the Meridians were 			stretched apart at the Poles to the same distance they are apart at 			the Equator. This causes extreme exaggeration of longitude in 			northerly areas. The Mercator Projection is relatively precise in 			depiction of distances in the equatorial regions. However, 				distortion, as has been stated, becomes more pronounced with 			distance from the Equator.

c.	To overcome this problem, the Mercator technique can be applied 			by rotating the cylinder 90 degrees so that the point of tangency is 			a meridian of longitude rather than the Equator. In this case, the 			chart is accurate along the selected meridian. Such a projection is 			called a Transverse Mercator Projection.

OHP #3

d.	The Transverse Mercator Projection, because of its method of			production, is quite accurate in depicting scale, especially on charts 		covering a relatively small geographical area.  Depending on the 			area to be depicted, any one of the 360 meridians of longitude can 			be selected as the point of tangency for the chart projection.

CONFIRMATION STAGE 1

1.	What are maps used for air navigation called?

Answer:	Charts

2.	What are the two principle types of projection used in air navigation 		charts?

Answer:	a.	Lambert Conformal Conic Projection

b.	Transverse Mercator Projection

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details - 	Each cadet will have to identify different types of projections used in air				navigation charts.

1.	Test knowledge with OHP #4.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you have learned about navigation charts.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You will have an opportunity to use this knowledge during the last,				Navigation class, where you will plot a flight plan.

C.	Your next lesson is PO/EO 417.02b. In this class you will learn more about navigation charts.