ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL FOUR | PO: NAVIGATION |
| --- | --- |

PO/EO:	417.01a

ENABLING OBJECTIVE:	Review Level Three Navigation.

REFERENCE(S):	A.	A-CR-CCP-269/PH-001 Level Four Course Training Plan

Chapter 4, Pages 82.

B.	A-CR-CCP-268/PT-001 Level Three Handbook

Chapter 10.

C.	A-CR-CCP-263/PT-001 From the Ground Up

Pages 164-167.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP Slides

LEARNING AID(S):		A.	A-CR-CCP-268/PT-001 Level Three Handbook

TEACHING POINT(S):	A.	The earth’s magnetic field

B.	The magnetic pole vs. the true pole

C.	Variation

D.	Isogonic lines

E.	Agonic lines

F.	Longitude

G.	Latitude													H.	The compass rose

J.	Magnetic north vs. true north

K.	Meridian lines

METHOD OF INSTRUCTION:	Review

TIME:		1 of 2 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL FOUR | PO: NAVIGATION |
| --- | --- |

PO/EO:	417.01a

ENABLING OBJECTIVE:	Review Level Three Navigation.

REVIEW

PO/EO:	N/A

ENABLING OBJECTIVE:		N/A

INTRODUCTION

WHAT:	In this lesson you will review Level Three Navigation.

WHY:	Knowledge of technical navigation procedures, as they apply to pilots, will help cadets understand the importance of navigation principles in plotting flights. Cadets must understand Level 3 Navigation thoroughly before they can move on to Level Four Navigation.

WHERE:	You will have an opportunity to use this knowledge during the last navigation class, where you will plot a flight plan.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	LEVEL THREE NAVIGATION REVIEW

5

MINs

1.	Review Questions:

a.	Describe the earth’s magnetic field.

Answer:	The Earth is magnetic and therefore has a North and

OHP #1

## South Magnetic Pole. Lines of magnetic force flow

between these two poles creating a magnetic field

that surrounds the Earth.

OHP #2

b.	Identify the difference between the magnetic pole and the true pole.

Answer:	The Earth makes one rotation each day about an

imaginary line called the spin axis. The two points at

which the spin axis terminates on the surface of the

Earth are called the North and South Poles. The

North Pole is often referred to as geographic north or

OHP #1

true north. The Earth is also magnetic and therefore

has a North and South Magnetic Pole. The Magnetic

North Pole is often referred to as magnetic north.

Unfortunately, the magnetic poles do not coincide with

the geographic poles.

c.	What is variation?

Answer:	The angle between the true meridian and the

OHP #3

magnetic meridian is called variation. Therefore, in

the Northern Hemisphere, variation is the angle

between true north and magnetic north. This angle

will be different depending on your position with

reference to the two meridians. Magnetic meridians

do not coincide with true meridians because magnetic

north does not coincide with true north. Therefore, if

magnetic north lies west of true north from a given

point, the compass needle will point west of true

north. This is because the magnetic meridian will also

lie west of the true meridian.

d.	What are isogonic lines?

Answer:	Lines drawn on a chart joining places having the

OHP #3

same variation are called isogonic lines or isogonals.

Isogonic lines are not straight but bend and twist due

to the influence on the Earth’s magnetic field by local

magnetic bodies below the surface of the Earth.

Isogonic lines are numbered east and west according

to whether the variation is to the east or west of true

north.

17

MINs

e.	What are agonic lines?

Answer:	There are places where true north and magnetic north

OHP #3

lie in the same straight line.  These places will,

therefore, have no variation.  Lines drawn on a chart

joining places with no variation are called agonic

lines.  Like isogonic lines, they twist and curve due to

the attraction of local magnetic bodies in the Earth.

f.	Describe meridians of longitude.

Answer:	Meridians of longitude circle the earth north to south

OHP #4

and converge at the true north and south poles. There

are 360 meridians of longitude. Longitude is

measured from 0 to 180 degrees east and west of the

prime meridian, which is numbered 0 degrees.

Longitude is expressed in degrees, minutes and

seconds.

g.	Describe parallels of latitude.

OHP #5

Answer:	Lines of latitude circle the earth east to west and do

not converge. There are 180 parallels of latitude.

Latitude is measured from 0 to 90 degrees north or

south of the equator, which is numbered 0 degrees.

Latitude is expressed in degrees, minutes and

seconds.

h.	What is a compass rose?

OHP #6

Answer:	Direction is measured clockwise from north to the

nearest degree and is expressed as a three-figure

group. Degrees are defined on the compass rose,

which is divided clockwise from north into 360

degrees. North is given 000 (often 360 is used) and

therefore, east, which is 90 degrees from north is 090,

south is 180 and west is 270.

j.	What is the difference between magnetic and true north?

Answer:	Unfortunately, the magnetic poles do not coincide with

the geographic poles.  Furthermore, magnetic north

changes position a little every year. Averaged over

thousands of years, the position of magnetic north will

roughly correspond with true north, but at any given

time the two poles can be very far apart.

k.	How do pilots use meridian lines in order to determine an aircraft’s			direction?

Answer:	The angle between the meridian over which an

aircraft is flying and the line in which the aircraft’s

nose is pointing, measured clockwise, is the true

heading.

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details - 	Each cadet will have to demonstrate an adequate knowledge of Level Three

Navigation.

1.	Test knowledge with the above questions.

2.	Provide assistance to the cadets as required to ensure they understand the material.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you reviewed Level Three Navigation.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You will have an opportunity to use this knowledge during the last				Navigation class, where you will plot a flight plan.

C.	Your next lesson is PO/EO 417.01b. In this class you will continue to review Level Three Navigation.